Installation notes for Windows NT 3.5x

Installation for Windows NT is in two parts:

(A). Installation of the drivers

(B). Installation of the applications

A - INSTALLATION OF THE DRIVERS

(1).  Start Windows NT 3.5x

(2).  Put DISK-1 into your floppy drive

(3).  Open the "Control Panel" group

(4).  Open the "Drivers" icon

(5).  Highlight "Unlisted or Updated Driver" then select "OK"

(6).  Type "A:\NT35"

(substitute proper drive for "A:" if necessary)

(7).  The sound card should already be highlighted, select "OK"

(8).  A message will pop up:

The required midimap.cfg is already on the system

do you want to use the current driver or install

a new driver?

Select "New"

(9).  Base I/O 220 will be displayed, select "Continue"

(10). Next display will show:

IRQ  7

DMA  1

MPU401 I/O Address  300

Change the MPU401 address to 330, then select "OK"

(11). Yow will need to shut-down and re-start Windows

for the driver installation to be complete.

B - INSTALLATION OF THE APPLICATIONS

First install the drivers, shut-down and restart Windows

(1).  Start Windows

(2).  Put DISK-2 into your floppy drive

(3).  Select "File" then "Run" from the program manager menu bar

(4).  Type "A:\WINAPPS", then select "OK"

(5).  The installtion will create a sound card group.

Within this group will be an icon for a "Midi Player".

You must change the properties of the "Midi Player" as follows:

(a). Highlight the icon for the "Midi Player"

(b). Select "File" then "Properties" from the program manager menu

(c). Change the command line

replace "\mplayer"

with "\system32\mplay32"

Be sure to leave the text before and after the same.

(d). Select "OK"

If you have the WaveTable option, you can change from the FM synthesizer

to the WaveTable synthesizer using the Control Panel.

(1). Open the "Control Panel"

(2). Open "Midi Mapper" in the Control Panel

(3). Select FM or WaveTable

(5). Select "Close"

You will have to stop and re-start the song playing for the change

to take effect.