Trademarks

NetWare and Novell are registered trademarks of Novell, Inc.

IBM is a registered trademark of IBM, Inc.

References

The following publications contain information that may be helpful

in reading this document:

1) NOVELL NetWare Version 3.11 Installation

(See Appendix D: Using DOSGEN to Add Remote Reset)

2) IBM Local Area Network Technical Reference, SC30-3383-3

3) IBM Remote Program Load User's Guide, 83X7840

4) IBM Token-Ring Network Architecture Reference, SC30-3374-02

Introducing RPL.com

RPL.com is a DOS Terminate and Stay Resident (TSR) Module that acts

as a Protocol Stack and responds to the IBM architected Remote

Program Load (RPL) frames as defined in the "IBM Remote Program

Load User's Guide".  It is used in networks that have diskless

workstations installed with the RPL BIOS Module.  Currently, this

is supported on the following network adapters:

IBM Ethernet Adapters

IBM PC Network Adapters

IBM Token-Ring Network Adapters

Specifically, RPL.com will respond to the following frames:

FIND                RPL.com will respond with a FOUND frame.

SEND.FILE.REQUEST   RPL.com will respond with a

FILE.DATA.RESPONSE frame.

RPL.com is a Terminate Stay Resident program to be loaded at Novell

DOS Clients that are also to function as "RPL Servers".  A Novell

ODI driver, configured for the IEEE 802.2 Frame type, must be loaded

at the Novell DOS Client "RPL Server" before RPL.com will operate

properly.

In addition, the BOOTNCP.com program must be loaded on the "RPL Server"

in order for it to respond to NetWare Core Protocol (NCP) requests made

by the bootstrap programs.

Features of RPL.com

RPL.com comes with five new and improved BOOTSTRAP programs:

ETHER.rpl, F1ETH.rpl, PCN2L.rpl, RBOOT.rpl, and TOKEN.rpl.

Together, these files offer the following features:

BOOTCONF.sys        Wildcard characters (* and ?) are allowed

in specifying Node Addresses.  Also, more

than one Disk Image File name is allowed

per Node Address.  BOOTCONF.sys is parsed

by RPL.com at the "RPL Server" to minimize

Network Traffic.

IBM LAN Support     RPL now works reliably with the IBM

LAN Support Program.

RPLFIX and RPLODI   These two programs are NOT required when

booting a workstation with an RPL style

prom installed.

Source Routing      Booting an RPL Client across a Source Routing

Bridge is supported..

PS/2 REFERENCE      The *.rpl files will communicate with the

BIOS on a PS/2 machine and ask it if it needs

to run REFERENCE.  If the BIOS says "yes" to

this query, RBOOT.RPL, ETHER.RPL, TOKEN.RPL,

F1ETH.RPL, AND PCN2L.RPL will automatically

present a menu on the workstation screen

with a "MXX.REF" boot image file name, where

"XX" is the PS/2 Model number returned by

the PS/2's BIOS.  This file name is for

the reference diskette image that needs to

be dosgened and copied into the \LOGIN

directory.  An example of one of these file

names would be:  MF8.REF where F8 is the model

number returned by a PS/2 Model 56LS BIOS.

Installing RPL.com

The following BOOTSTRAP programs come with RPL.com and should be

installed in a  \LOGIN  directory on the RPL Server.

ETHER.rpl      For IBM MCA Ethernet Adapters

(an SMC/WD 8003/A adapter OEM'd by IBM)

F1ETH.rpl      For IBM Model 25SX Ethernet Adapters

(an ISA DLINK ethernet adapter OEM'd by IBM)

PCN2L.rpl      For IBM PC Network Adapters

RBOOT.rpl      For Adapters using the Novell Enhanced Boot Proms

(proms engineered to the Novell Boot ROM spec.)

TOKEN.rpl      For IBM Shared-Ram Token-Ring Network Adapters

An appropriate BOOTSTRAP program gets downloaded to the workstation

in response to a SEND.FILE.REQUEST from the Boot ROM RPL Module.

RPL.com should be installed on the Hard Disk of the RPL Server.

Loading RPL.com on the RPL Server

To use RPL.com, you must have installed the RPL ROM Module on your

network adapter board in the RPL Client.  This RPL module must be

capable of sending the IBM architected RPL frame sequence.  See the

"IBM Remote Program Load User's Guide" for information on this

architecture.

Implementing the RPL function consists of creating a Disk Image

File in the \LOGIN directory of the drive where RPL.com is loaded.

A description of this process is given in the NOVELL NetWare

Version 3.11 Installation manual.

In addition,  certain IBM Micro-Channel computers, such as the new

386SLC, have a BIOS image file associated with them.  This file

comes with the computer on a diskette, and has a file extension of

.IML.  You MUST create a directory called \LOGIN\IBMLAN\DCDB\IMAGES

and install ALL .IML files in this directory.

BOOTNCP.com must be loaded at the DOS command prompt (or from a batch

file) before RPL.com is loaded as follows:

BOOTNCP <ENTER>

BOOTNCP.com is a program written for the RPL Server that responds to

NetWare Core Protocol (NCP) frames for accessing the Net$dos.sys file

or other boot image files.

RPL.com can then be loaded from the DOS command promt or from a batch

file.  RPL.com syntax or command line parameters follow:

RPL  [ACK] [BIND board] [BUFFERS=nn] [DEFAULT] [FRAME=ff]

[GNS] [NOACK] [NODEFAULT] [NOGNS] [NOPROTECT] [NOTRO]

[PROTECT] [PS=server] [TRO] [U] [UNBIND board]

[WAIT TIME=ssss] [?] <ENTER>

RPL.com command line parameters explained in detail:

The parameters specified by "[]" (e.g. [ACK] etc.) are OPTIONAL,

NOT Case Sensitive, separated by blanks or commas, and may be

entered in any order.  They are described below:

ACK       Use this parameter if you wish to configure the RPL

BOOT ROM Module to ACKnowledge FILE.DATA.RESPONSE

frames sent by RPL.com.  By DEFAULT, RPL.com will

send FILE.DATA.RESPONSE frames in a BURST mode.

This parameter allows pacing by the workstation, if

the Adapter on the workstation cannot keep up with

RPL.com.

BIND      Use this parameter to BIND RPL to a specific

NetWare ODI board that is configured for the IEEE

802.2 frame type.  board may be specified by name,

or optionally, by MLID board number (eg BIND #n),

where n is a DECIMAL number.

If multiple boards are to be BOUND; specify the

BIND parameter MULTIPLE times.

If BIND is not specified on the Command Line or in

the NET.cfg file, RPL.com will search for the FIRST

802.2 board that is installed and use it.

BUFFERS=nn     nn is a DECIMAL number specifying the number

of Receive buffers to configure.  The DEFAULT

is FIVE.

DEFAULT   This parameter will OVERRIDE the NODEFAULT

parameter if it is specified on the BIND command in

NET.cfg.

FRAME=ff  Use this parameter if you wish to configure the

BOOTSTRAP program to use a frame type other than

the default to access the file server.  Valid

responses are:

FRAME=802.2    This is the DEFAULT.

FRAME=EII      Configure the BOOTSTRAP program for

ETHERNET_II.  This option should

only be used on ETHERNET networks.

FRAME=SNAP     Configure the BOOTSTRAP program for

the SNAP frame type.

If FRAME=EII or FRAME=SNAP is selected, RPL.com

will force the BOOTSTRAP program to do a Get

Nearest Server (GNS).

The FRAME parameter is fully supported on BOOT ROMs

that use the RBOOT.rpl BOOTSTRAP program.  This

parameter is IGNORED on BOOT ROMs that use

PCN2L.rpl and TOKEN.rpl.  However, FRAME=EII or

FRAME=SNAP will configure ETHER.rpl to use

ETHERNET_II instead of 802.3.  Specifying FRAME=EII

on a Token-Ring BOOT ROM will be ignored.

GNS       This parameter specifies that you wish the

workstation to do a Get Nearest Server request when

the appropriate BOOTSTRAP program gets downloaded.

Normally, RPL.com will fill in the bootstrap

program with the RPL Server information, so that

it does NOT need to do a Get Nearest Server request.

Using this parameter MAY cause the RPL workstation

to find a server OTHER than the one where RPL.com

is located.

NOACK     This parameter will OVERRIDE the ACK parameter if

it is specified on the BIND command in NET.cfg.

NODEFAULT This parameter tells RPL.com to NOT respond to a

FIND frame UNLESS the node address of the

workstation is found in the BOOTCONF.sys file.  It

is provided for security reasons.  The workstation

WILL NOT boot until the system administrator

inserts into the BOOTCONF.sys file the node address

and associated Disk Image File name(s) to use when

booting the workstation.  A further description of

BOOTCONF.sys is given under the heading of Unique

Boot Sequences using RPL.com.

NOGNS     This parameter will OVERRIDE the GNS parameter if

it is specified on the BIND command in NET.cfg.

NOPROTECT This parameter will OVERRIDE the PROTECT parameter

if it is specified on the BIND command in NET.cfg.

NOTRO     This parameter will OVERRIDE the TRO parameter if

it is specified on the BIND command in NET.cfg.

PROTECT   This parameter tells RPL.com to configure the

BOOTSTRAP program so that it will protect itself in

the workstation memory.  It does this by adjusting

the Memory Size variable in the BIOS data area

(40:13) to reflect the amount of memory that it

uses.  Using this parameter will REDUCE the amount

of memory that the workstation has available for

DOS by about 12k.  It is recommended that this

parameter NOT be USED unless absolutely necessary.

PS=server This parameter tells the BOOTSTRAP program to

attach to a NetWare file server other than the one

where RPL.com is loaded.  server is the name of a

NetWare file server that contains the Image file

for the workstation.

TRO       This parameter specifies that you wish the

Bootstrap Program to do a This Ring Only count of

THREE (3) on all Broadcasts frames.  It is useful

in a SOURCE ROUTING environment and Servers are

available on the local ring.

U         This parameter will UNLOAD a previously installed

RPL.com.  All boards that are BOUND will be

UNBOUND,  and all memory used returned to DOS.

UNBIND    Use this parameter to UNBIND RPL from a specific

NetWare ODI board.  board may be specified by name,

or optionally, by MLID board number (eg UNBIND #n),

where n is a DECIMAL number.

WAIT TIME=ssss This parameter specifies the number of SECONDS

you wish the workstation to wait before

automatically selecting the cursored Disk

Image Name when multiple Disk Image Names are

specified in BOOTCONF.sys.  (See "BOOTCONF.sys

Extensions")

The DEFAULT for this parameter is 0000, which

means forever.  Any value from 0000 to 65535

is allowed.

?         This parameter will display a list of all boards

which RPL.com is currently BOUND.

Creating and using NET.cfg

The NET.cfg file is used by various ODI modules (including the LSL,

LAN Drivers, and Protocol Stacks) to obtain the network system

configuration information at initialization time.  RPL.com reads

this file and parses it for a section describing the RPL

configuration to use.  Specifically, it searches for the following

MAIN section heading:

PROTOCOL RPL

Note that the heading is NOT case sensitive, but the word

"PROTOCOL" MUST begin in column one (1) of the file.

After this heading is found, RPL.com looks for the following

INDENTED Keywords to configure itself:

PROTOCOL RPL

BIND board [ACK] [GNS] [NODEFAULT] [PS=server]

[PROTECT] [TRO] [WAIT TIME=ssss]

BUFFERS nn

The parameters specified by "[]" are OPTIONAL, NOT Case Sensitive,

separated by blanks, and may be entered in any order. They are

described below:

BUFFERS=nn     nn is a DECIMAL number specifying the number

of Receive buffers to configure.  The DEFAULT

is FIVE.  Use this parameter to optimize for

performance.

BIND      Use this parameter to BIND RPL to a specific

NetWare ODI board that is configured for the IEEE

802.2 frame type.  board may be specified by name,

or optionally, by MLID board number (eg BIND #n),

where n is a DECIMAL number specifying the board to

BIND.

If multiple boards are to be BOUND; specify the

BIND parameter MULTIPLE times, each on a DIFFERENT

line in NET.cfg.

If BIND is not specified on the Command Line or in

the NET.cfg file, RPL.com will search for the FIRST

802.2 board that is installed and use it.

The parameters specified with the BIND command are

OPTIONAL and are described below:

ACK       Use this parameter if you wish to configure

the RPL BOOT ROM Module to ACKnowledge

FILE.DATA.RESPONSE frames sent by RPL.com.  By

DEFAULT, RPL.com will send FILE.DATA.RESPONSE

frames in a BURST mode.  This parameter allows

pacing by the workstation, if the Adapter on

the workstation cannot keep up with RPL.com.

FRAME=ff  Use this parameter if you wish to configure

the BOOTSTRAP program to use a frame type

other than the default to access the file

server.  Valid responses are:

FRAME=802.2    This is the DEFAULT.

FRAME=EII      Configure the BOOTSTRAP program

for ETHERNET_II.  This option

should only be used on ETHERNET

networks.

FRAME=SNAP     Configure the BOOTSTRAP program

for SNAP.

If FRAME=EII or FRAME=SNAP is selected,

RPL.com  will force the BOOTSTRAP program to

do a Get Nearest Server (GNS).

The FRAME parameter is fully supported on BOOT

ROMs that use the RBOOT.rpl BOOTSTRAP program.

This parameter is IGNORED on BOOT ROMs that

use PCN2L.rpl and TOKEN.rpl.  However,

FRAME=EII or FRAME=SNAP will configure

ETHER.rpl to use ETHERNET_II instead of 802.3.

Specifying FRAME=EII on a Token-Ring BOOT ROM

will be ignored.

GNS       This parameter specifies that you wish the

workstation to do a Get Nearest Server request

when the appropriate BOOTSTRAP program gets

downloaded.  Normally, RPL.com will fill in

the bootstrap program with the RPL Server

information, so that it does NOT need to do

a Get Nearest Server request.  Using this

parameter MAY cause the workstation to find a

server OTHER than the one where RPL.com

is located.

NODEFAULT This parameter tells RPL.com to NOT respond to

a FIND frame UNLESS the node address of the

workstation is found in the BOOTCONF.sys file.

It is provided for security reasons.  The

workstation WILL NOT boot until the system

administrator inserts into the BOOTCONF.sys

file the node address and associated Disk

Image File name(s) to use when booting the

workstation.  A further description of

BOOTCONF.sys is given under the heading of

Unique Boot Sequences using RPL.com.

PROTECT   This parameter tells RPL.com to configure the

BOOTSTRAP program so that it will protect

itself in the workstation memory.  It does

this by adjusting the Memory Size variable in

the BIOS data area (40:13) to reflect the

amount of memory that it uses.  Using this

parameter will REDUCE the amount of memory

that the workstation has available for DOS by

about 12k.  It is recommended that this

parameter NOT be USED unless absolutely

necessary.

PS=server This parameter tells the BOOTSTRAP program to

attach to a NetWare file server other than the

one where RPL.com is loaded.  server is the

name of a NetWare file server that contains

the Image file for the workstation.

TRO       This parameter specifies that you wish the

Bootstrap Program to do a This Ring Only count

of THREE (3) on all Broadcasts frames.  It is

useful in a SOURCE ROUTING environment and

Servers are available on the local ring.

WAIT TIME=ssss This parameter specifies the number of

SECONDS you wish the workstation to wait

before automatically selecting the

cursored Disk Image Name when multiple

Disk Image Names are specified in

BOOTCONF.sys.  (See "BOOTCONF.sys

Extensions")

The DEFAULT for this parameter is 0000,

which means forever.  Any value from 0000

to 65535 is allowed.

Unique Boot Sequences using RPL.com

BOOTCONF.sys is an ASCII text file that allows you to specify a

unique Disk Image File for each workstation that needs access to

different files.  You create BOOTCONF.sys using your favorite DOS

ASCII text editor.  The process of creating unique Disk Image

Files is described in detail in the NOVELL NetWare Version 3.11

Installation manual.

BOOTCONF.sys Extensions

When RPL.com loads it will search the \LOGIN directory of the

current drive for a BOOTCONF.sys file.  If it finds it, it will

read it into a memory buffer so that it can parse it when a FIND

frame is received from a workstation.  Note that the parsing of

BOOTCONF.sys is done by RPL.com and NOT the BOOTSTRAP program to

minimize the amount of traffic on the network during the RPL

process.  The extensions to BOOTCONF.sys are given in this section.

Using Wildcard Characters in BOOTCONF.sys

Wildcard characters (* and ?) are allowed in the line specifying

the node address of the workstation.  This will allow the system

administrator more flexibility in building the BOOTCONF.sys file.

The rules for these wildcard characters are:

*    Use the ASTERISK character to specify a RANGE of digits

in the node address. For example, if the node address of

the workstation is 10005A123456, it may be specified as

0x*123456 in BOOTCONF.sys.  In this example, RPL.com will

match the node address with ANY node address that ends in

123456.

Note that only ONE asterisk (*) may appear in the node

address.

?    Use the QUESTION MARK character to specify ANY digit in

the node address.  In the example above, the node address

could be specified as 0x??????123456 which is equivalent

to 0x*123456.

You may use wildcard characters to specify a DEFAULT Disk

Image File for all workstations on the network that do

not have a specific Disk Image File.  You do this by

placing the line:

0x* = DEFAULT.sys

or        0x???????????? = DEFAULT.sys

as the LAST line in BOOTCONF.sys.  Either one of these

lines will match on ALL workstation node addresses.  The

DEFAULT.sys (or any name you desire) Disk Image File is

generated by DOSGEN, the same as any Disk Image File.

More than ONE Disk Image File per Node Address

Each line in BOOTCONF.sys that contains a node address may specify

more than one Disk Image File name, separated by one or more BLANK

characters.  In this case, the BOOTSTRAP program will present the

workstation user a PROMPT at boot up time to select the Disk Image

File to boot.  For example,  if a workstation's node address is

10005A123456, the line:

0x10005a123456 = ONE.sys TWO.sys THREE.dos

will cause the BOOTSTRAP program to present:

Place CURSOR on DISK IMAGE file; Hit ENTER when Ready:

ONE.sys

TWO.sys

THREE.dos

on the workstation screen.  The BOOTSTRAP program will then

use NCP calls to open the selected Disk Image File.  If it

does NOT exist, it will display:

Unable to OPEN Disk Image File

and retry the operation.

Up to TEN Disk Image File names may be entered for each node

address in BOOTCONF.sys.  Note that they must be separated by

one or more BLANK characters, and they must all fit on ONE

line.

Multiple Lines per Node Address

The ASCII COLON (:) can be used to allow for multiple lines when

specifying a particular node address in BOOTCONF.sys.  It is

provided for convenience when specifying multiple parameters on the

Node Address line.

To use this feature, place the ASCII COLON (:) at the end of the

line.  Note that it must be preceded by at LEAST one ASCII blank:

0x10005a460025 = NET$DOS.sys FRED.sys :

JOE.sys

BOOTCONF.sys BIND Override Parameters

When RPL.com parses BOOTCONF.sys, it allows the user to override

the BIND time parameters with parameters specific to a particular

workstation that is being booted.  By default, the parameters

entered on the Command Line when RPL.com was loaded will be merged

with the parameters that were given by the BIND keyword in NET.cfg

to apply to ALL workstations that are attached to the particular

board RPL.com is BOUND to.  The following commands are allowed

on a per NODE basis, which if used, will override ALL previous

parameters given:

ACK       Use this parameter if you wish to configure the RPL

BOOT ROM Module program to ACKnowledge

FILE.DATA.RESPONSE frames sent by RPL.com.  By

DEFAULT, RPL.com will send FILE.DATA.RESPONSE

frames in a BURST mode.  This parameter allows

pacing by the workstation, if the Adapter on the

workstation cannot keep up with RPL.com.

FRAME=ff  Use this parameter if you wish to configure the

BOOTSTRAP program to use a frame type other than

the default to access the file server.  Valid

responses are:

FRAME=802.2    This is the DEFAULT.

FRAME=EII      Configure the BOOTSTRAP program for

ETHERNET_II.  This option should

only be used on ETHERNET networks.

FRAME=SNAP     Configure the BOOTSTRAP program for

SNAP.

If FRAME=EII or FRAME=SNAP is selected, RPL.com

will force the BOOTSTRAP program to do a Get

Nearest Server (GNS).

The FRAME parameter is fully supported on BOOT ROMs

that use the RBOOT.rpl BOOTSTRAP program.  This

parameter is IGNORED on BOOT ROMs that use

PCN2L.rpl and TOKEN.rpl.  However, FRAME=EII or

FRAME=SNAP will configure ETHER.rpl to use

ETHERNET_II instead of 802.3.  Specifying FRAME=EII

on a Token-Ring BOOT ROM will be ignored.

GNS       This parameter specifies that you wish the

workstation to do a Get Nearest Server request when

the appropriate BOOTSTRAP program gets downloaded.

Normally, RPL.com will fill in the bootstrap

program with the RPL Server's information,

so that the bootstrap program does NOT need to do a

Get Nearest Server request.  Using this parameter

MAY cause the workstation to find a server OTHER

than the one where RPL.com is located.

NOACK     This parameter will override the ACK parameter

specified on the BIND command.

NOGNS     This parameter will override the GNS parameter

specified on the BIND command.

NOPROTECT This parameter will override the PROTECT parameter

specified on the BIND command.

NOTRO     This parameter will OVERRIDE the TRO parameter if

it is specified on the BIND command in NET.cfg.

PROTECT   This parameter tells RPL.com to configure the

BOOTSTRAP program so that it will protect itself in

the workstation memory.  It does this by adjusting

the Memory Size variable in the BIOS data area

(40:13) to reflect the amount of memory that it

uses.  Using this parameter will REDUCE the amount

of memory that the workstation has available for

DOS by about 12k.  It is recommended that this

parameter NOT be USED unless absolutely necessary.

PS=server This parameter tells the BOOTSTRAP program to

attach to a NetWare Server other than the RPL Server

where RPL.com is loaded.  server is the name of a

NetWare file server that contains the Image file

for the workstation.

REP       string1|string2 allows you to REPlace ALL

occurrences of string1 with string2 in the Disk

Image File.  The '|' (ASCII 7Ch) delimiter is

REQUIRED to delimit the string values.

Use this parameter to DYNAMICALLY re-configure a

Disk Image File during the RPL process.  It is

useful for tailoring files such as AUTOEXEC.BAT or

CONFIG.SYS to a specific workstation.

The rules for using REP are given below:

1)   The search IS case sensitive.  The Bootstrap

program will search for string1 EXACTLY as it

is entered in BOOTCONF.sys.

2)   ALL occurrences of string1 will be replaced

with string2 in the Disk Image File.

3)   string2 MUST be equal to OR shorter than

string1.

4)   If string2 is shorter than string1, the Disk

Image File will be padded with ASCII Blanks

when the substitution is made.

5)   string2 must contain NO embedded ASCII Blanks.

The FIRST blank that is encountered is

interpreted as the end of the string.

TRO       This parameter specifies that you wish the

Bootstrap Program to do a This Ring Only count of

THREE (3) on all Broadcasts frames.  It is useful

in a SOURCE ROUTING environment and Servers are

available on the local ring.

WAIT TIME=ssss This parameter specifies the number of SECONDS

you wish the workstation to wait before

automatically selecting the cursored Disk

Image Name when multiple Disk Image Names are

specified in BOOTCONF.sys.  (See "BOOTCONF.sys

Extensions")

The DEFAULT for this parameter is 0000, which

means forever.  Any value from 0000 to 65535

is allowed.

RPL.com parses the Node Address line of BOOTCONF.sys looking for

these keywords.  If an entry if found, but does not match one of

the keywords, it is assumed to be a Disk Image File name.

Therefore, you should NOT have a Disk Image File named the same as

any of these keywords.  Note that these parameters are OPTIONAL,

NOT Case Sensitive, separated by blanks, and may be entered in any

order.

An example of a BOOTCONF.sys line using these parameters is:

0x*1234 = NET$DOS.sys gns protect rep NODE=^^^^^|NODE=67890

In this case, gns and protect will be interpreted as KEYWORDS and

not Disk Image File names. In addition, the string 'NODE=^^^^^'

will be REPLACED with 'NODE=67890' wherever it occurs in the Disk

Image File.

Changing BOOTCONF.sys

RPL.com reads BOOTCONF.sys into a memory buffer at LOAD time.  If

the user changes BOOTCONF.sys after  RPL.com is loaded, it must be

unloaded by specifying:

RPL u

on the DOS command line.  When RPL.com is loaded again, it will

read the new copy of BOOTCONF.sys.

Memory Considerations

RPL.com will read ALL Bootstrap programs with a .RPL extension it

finds in the \LOGIN directory and caches them into memory when it

loads.  It is designed to provide multivendor support with any and

all boards that use the IBM RPL Architecture.  Each Bootstrap

program consumes anywhere from 10 to 15 Kilobytes of memory.  To

reduce the amount of RESIDENT memory RPL.com consumes, it is

suggested you load ONLY the Bootstrap programs that will actually

be used into the RPL Server's \LOGIN directory.

BOOTCONF.sys is the only other file that is cached during

initialization time.  This file is OPTIONAL, so if it is NOT needed

there is no reason to define it.  It does, however, offer some

powerful configuration options, so when and if you do define it, it

is suggested that it be made as small as possible.