Installation notes for OS/2 ver 3.0 (Warp).

OS/2 Installation will do the following:

A. Allow OS/2 system sounds (starting OS/2, opening folders, etc)

B. Allow OS/2 applications to play sounds (MultiMedia, games, etc)

C. Creates an Audio player in the MultiMedia folder

for playing digitized sounds

D. Creates a MIDI player in the MultiMedia folder

for playing Midi music with the FM synthesizer

E. Creates a MIDI:2 player in the MultiMedia folder

for playing Midi music with the WaveTable synthesizer

Installation for OS/2 is in three parts:

A. Installing OS/2 MMPM/2

(you can skip this step if it is already installed)

B. Installing OS/2 device drivers

a). Installing Digitized sounds and FM synthesis

b). Installing WaveTable synthesis (requires WaveTable option)

C. Installing WIN-OS/2 device drivers

(you can skip this step if you do not run WIN-OS/2)

A. INSTALLING OS/2 MMPM/2

Note:

To install OS/2 Warp sound support you must have the OS/2 MMPM/2 support

installed.  MMPM/2 is normally installed automatically with OS/2 unless

you specifically chose not to install it.  If you have a folder on your

desktop called "Multimedia" then you have MMPM/2 support and you can

proceed to Step B - Installing OS/2 device drivers.  If you don't

have this folder then you will have to install MMPM/2 support.  The

following steps will guide you thru installing MMPM/2:

(1).  Boot OS/2 Warp

(2).  Select the "OS/2 System" icon

(3).  Select "System Setup" icon

(4).  Select "Selective Install" icon

(5).  Select "MultiMedia Device Support" button

(6).  You should see a list of devices in the left column and no

sound card in the right hand column.  If there is a sound card

listed in the right hand column, highlight the sound card and

select "<< REMOVE".

Note:

OS/2 comes with a SoundBlaster PRO MCV device driver.  However,

the SoundBlaster drivers that come with OS/2 only support 8-bit audio.

Do Not Install the SoundBlaster drivers.

Although these drivers will work fine, the sound card is capable of

much higher quality 16-bit audio.  After MMPM/2 support is installed,

you will add the sound card's 16-bit audio support to OS/2.

(8).  Select "OK" to return to System Configuration menu

(9).  Select "OK" to return to OS/2 Setup and Installation menu

The Multimedia Software Support checkbox should be checked.  If it is

not, then use your mouse and click on the box to check it.

(10). Select "Install" to begin the install process

(11). After Installation is complete, you will have to Shutdown and then

restart OS/2.

B. INSTALLING OS/2 DEVICE DRIVERS

Note: You must have MMPM/2 support loaded.

Read the previous section about installing MMPM/2.

(1).  Start OS/2 Warp

(2).  Put the sound card's DISK-1 into your floppy drive

(3).  Open the Multimedia folder on your desktop

(4).  Find the icon for the Multimedia Applications Install and double

click on this icon with your mouse.

(5).  Select your floppy drive for the source drive

(6).  Select the "OS2v3" directory on the sound card's floppy disk

(7).  You have two choices:

"Sound" support for digital sounds and FM synthesis

"WaveTable" support for WaveTable synthesis

(8).  The "Sound" support is automatically checked (selected to install)

Use your mouse and click on the "WaveTable" support if you have

the "WaveTable" option.

(9).  Select "Install" to start the installation from the floppy disk

(10).  When asked to select the total number of sound cards in the system

select 1 and then select "OK".  If you selected "Sound" and WaveTable"

support then you will be asked twice.

(11).  Select "OK" to automatically update your CONFIG.SYS file

(12).  Select "OK" from the Congratulations message

(13).  Remove the floppy from the drive

(14).  Shutdown and restart OS/2.

If you have the WaveTable option (and installed "WaveTable" support),

you will have two MIDI players in your MultiMedia folder:

MIDI player is for FM

MIDI:2 player is for WaveTable

You can change the names of these players to "FM MIDI" and "WaveTable MIDI"

if you desire using the settings for each object.

C. INSTALLING WIN-OS/2 DEVICE DRIVERS

(1).  Start a full screen Win-OS/2 session

(2).  Put DISK-1 into your floppy drive

(3).  Select "File" and then "Run" from the program manager

(4).  Type "A:SETUP"

(substitute proper drive for "A:" if necessary)

(5).  After copying files to your hard disk, the installation program

will switch to the OS/2 desktop.  At this point use "CTRL-ESC" to

bring up the task list and switch back to the Win-OS/2 Full Screen

Session where you can finish the installation.

Be sure to read the document file at the end of installation

You will need to exit and re-start Win-OS/2

for the install to be complete.

If you have the WaveTable option, you can change from the FM synthesizer

to the WaveTable synthesizer using the Win-OS/2 Control Panel.

(1). Open the "Control Panel"

(2). Open "Midi Mapper" in the Control Panel

(3). Select FM or WaveTable

(5). Select "Close"

You will have to stop and re-start the song playing for the change

to take effect.