Date: 09/25/92

Status: Beta 2.0.06

Trident

TVGA 8900/9000 PM Display Driver Package

for OS/2 Version 2.0

IMPORTANT: Before installing any of the programs in this package, make

^^^^^^^^^  sure that you read through this document once.

Download Info

-------------

The drivers and utilities are compressed in serveral different

files.  Instead of downloading the all these files  listed below (which

might consume a lot of connection time), we recommend that you choose

those drivers that meet your current needs and system configuration.

After downloading any of the files in this package, please

use the PKUNZIP.EXE utility to unpack them.  All the files listed

below are available from Trident BBS:

README.EXE      - LHA self extractable README.DOC

*W800.ZIP        - W800.DLL (800x600-16 Seamless DISPLAY.DLL)

W800SL.DRV (Seamless SWINVGA.DRV)

W800.DRV (WinOS2 full screen VGA.DRV)

W1024.ZIP       - W1024.DLL (1024x768-16 DISPLAY.DLL)

W1024C.DRV (WinOS2 full screen VGA.DRV)

W8514.ZIP       - W8514.DLL (1024x768-16 DISPLAY.DLL with

8514 type fonts and icons)

W1024F.DRV (WinOS2 full screen VGA.DRV

with 8514 type fonts and icons)

T640.ZIP        - T640.DLL (640x480-256 DISPLAY.DLL)

T640C.DRV (WinOS2 full screen VGA.DRV)

T768.ZIP        - T768.DLL (768x1024-256 DISPLAY.DLL-portrait)

T800.ZIP        - T800.DLL (800x600-256 DISPLAY.DLL)

T800C.DRV (WinOS2 full screen VGA.DRV)

T1024.ZIP       - T1024.DLL (1024x768-256 DISPLAY.DLL)

T1024C.DRV (WinOS2 full screen VGA.DRV)

T8514.ZIP       - T8514.DLL (1024x768-256 DISPLAY.DLL with

8514 type fonts and icons)

T1024FC.DRV (WinOS2 full screen VGA.DRV

with 8514 type fonts and icons)

T640V512.ZIP    - T640V512.DLL (1024x512-256 Virtual Desktop

DISPLAY.DLL on 640x480 physical screen,

for 512 KB memory ONLY)

T640V.ZIP       - T640V.DLL (1024x1024 Virtual Desktop

DISPLAY.DLL on 640x480 physical screen,

requires 1MB or more)

T800V.ZIP       - T800V.DLL (1024x1024 Virtual Desktop

DISPLAY.DLL on 800x600 physical screen,

requires 1MB or more)

UTILS.ZIP       - DESKMAN.EXE: PM desktop manager (supports

zoomable virtual desktop)

DESKDLL.DLL: library used by DESKMAN.EXE

DISPINFO.EXE: shows WinOS2 display info

SMODE.EXE: set mode for both OS/2 and DOS

POLARITY.COM: DOS TSR to change 800x600-16

mode sync polarity for some of monitors

TRIDENT.EXE: identifies which type of

Trident TVGA controller is installed

FIXES.ZIP       - OS2LDR: OS/2 system loader fix

VSVGA.SYS: virtual VDD fix

SVGA.EXE: .PMI generator fix

PATCH256.EXE: patch desktop trash (256 color)

8900BFIX.COM: patch for some 8900B boards

(* from Beta 2.0.01. unmodified)

Please note, the files W1024C.DRV, T640C.DRV, T800C.DRV,

T1024C.DRV and T1024FC.DRV are for TVGA 8900C only.  Use your own

regular Windows 3.0 drivers (with your TVGA card) is preferred.

This package may also be available from the following

BBSes or Internet FTP archives (could be different file names):

IBM NSC BBS:    (404) 835-6600

Fernwood:       (203) 483-0348  (east cost)

OS/2 Shareware: (703) 385-4325

Greater Chicago:(708) 895-4042  (central)

Bay Area OS/2:  (510) 657-7948  (west cost)

CompuServe      IBMOS2 forum

ftp-os2.nmsu.edu                (anonymous FTP)

novell.com (not day time)       (anonymous FTP)

mtsg.ubc.ca                     (anonymous FTP)

archive.latrobe.edu.au          (anonymous FTP)

funic.funet.fi                  (anonymous FTP)

usenet news system: comp.os.os2.binaries

Newer releases of OS/2 drivers will be posted on Trident

BBS later.  Please check the date and status of the driver files.

Descriptions:

-------------

( The following descriptions assume that OS/2 2.0 is installed on

the target system, drive C:. )

The file W800.DLL is a customized PM display driver which supports

the 800x600-16 color mode PM Desktop session.  This file (if 800x600 16

color mode is chosen as the desired display mode) will replace the default

DISPLAY.DLL (640x480-16 color standard VGA mode) file in directory

C:\OS2\DLL.  This driver is fully compatible with 2.0 display driver spec

and is capable of running seamless Windows applications.  To run seamless

Windows apps on 800x600-16 workplace shell, you need also install

W800SL.DRV.  To run WinOS2 full screen in 800x600-16 mode, you have to

replace C:\OS2\MDOS\WINOS2\SYSTEM\VGA.DRV by W800.DRV.

The file W1024.DLL is a customized PM display driver which supports

the 1024x768-16 color mode PM workplace shell.  This file will replace the

default DISPLAY.DLL file in directory C:\OS2\DLL.  To run WinOS2 full

screen in 1024x768-16 mode, you have to replace C:\OS2\MDOS\WINOS2\SYSTEM

\VGA.DRV by W1024C.DRV.

The file W8514.DLL is similar to W1024.DLL, which supports the

1024x768-16 color mode PM workplace shell.  However, it contains 8514

compatible type of fonts and icons.  This file will replace the default

DISPLAY.DLL file in directory C:\OS2\DLL.  To run WinOS2 full screen

in 1024x768-16 mode using 8514 compatible type of fonts and icons,

you have to replace C:\OS2\MDOS\WINOS2\SYSTEM\VGA.DRV by W1024F.DRV.

The file T640.DLL is a customized PM display driver which supports

the 640x480-256 color mode PM workplace shell.  This file will replace the

default DISPLAY.DLL file in directory C:\OS2\DLL.  To run WinOS2 full

screen in 640x480-256 mode, you have to replace C:\OS2\MDOS\WINOS2\SYSTEM

\VGA.DRV by T640C.DRV.

The file T768.DLL is a customized PM display driver which supports

the 768x1024-256 (portrait mode) color mode PM workplace shell.  This file

will replace the default DISPLAY.DLL file in directory C:\OS2\DLL.  Special

SVGADATA.PMI parameters are required to run this mode.

The file T800.DLL is a customized PM display driver which supports

the 800x600-256 color mode PM workplace shell.  This file will replace the

default DISPLAY.DLL file in directory C:\OS2\DLL.  To run WinOS2 full

screen in 800x600-256 mode, you have to replace C:\OS2\MDOS\WINOS2\SYSTEM

\VGA.DRV by T800C.DRV.

The file T1024.DLL is a customized PM display driver which supports

the 1024x768-256 color mode PM workplace shell.  This file will replace the

default DISPLAY.DLL file in directory C:\OS2\DLL.  To run WinOS2 full

screen in 1024x768-256 mode, you have to replace C:\OS2\MDOS\WINOS2\SYSTEM

\VGA.DRV by T1024C.DRV.

The file T8514.DLL is similar to T1024.DLL, which supports the

1024x768-256 color mode PM workplace shell.  However, it contains 8514

compatible type of fonts and icons.  This file will replace the default

DISPLAY.DLL file in directory C:\OS2\DLL.  To run WinOS2 full screen

in 1024x768-256 mode using 8514 compatible type of fonts and icons,

you have to replace C:\OS2\MDOS\WINOS2\SYSTEM\VGA.DRV by T1024CF.DRV.

The file T640V512.DLL is a Virtual Desktop PM display driver which

supports 1024x512-256 color PM workplace shell through a physical 640x480

256 color display screen.  This file will replace the default DISPLAY.DLL

file in directory C:\OS2\DLL.  This driver supports 'zoomable' feature.

This driver should be used on TVGA boards with 512KB memory, since zoom

might not work correctly on 1MB boards.  (For more info on Virtual Desktop,

see Appendices below.)

The file T640V.DLL is a Virtual Desktop PM display driver which

supports 1024x1024-256 color PM workplace shell through a physical 640x480

256 color display screen.  This file will replace the default DISPLAY.DLL

file in directory C:\OS2\DLL.  This driver supports 'zoomable' feature.

(For more info on Virtual Desktop, see Appendices below.)

The file T800V.DLL is a Virtual Desktop PM display driver which

supports 1024x1024-256 color PM workplace shell through a physical 800x600

256 color display screen.  This file will replace the default DISPLAY.DLL

file in directory C:\OS2\DLL.  This driver supports 'zoomable' feature.

(For more info on Virtual Desktop, see Appendices below.)

The file W800SL.DRV (zipped in W800.ZIP) is a customized WINOS2

display driver which supports Windows applications running on PM workpalce

shell seamlessly.  The PM workplace shell should run in 800x600-16 mode

(using W800.DLL as DISPLAY.DLL).  This file will replace the default

SWINVGA.DRV in directory C:\OS2\DLL.

TRIDENT.EXE is a DOS utility which identifies the type of Trident

TVGA card on your system.  Run it under DOS full screen only.

DESKMAN.EXE is a PM program which provides display driver info

and virtual desktop control.  With this release, the virtual desktop

drivers supports zoom-in and scroll lock functions (see appendices for

detailed descriptions).

DESKDLL.DLL is the dyna-link library called by DESKMAN.EXE.

DISPINFO.EXE is a WinOS2 program which displays the WinOS2

display driver information.

SMODE.EXE is a set video mode utility for both OS/2 and DOS.

POLARITY.COM is a DOS TSR program to adjust DOS session 800x

600-16/256 mode horizontal and vertical polarity.  By default, video

BIOS set the horizontal and vertical polarities to both negative in

these two modes.  But some monitor requires positive vertical sync

polarity.  If you encounter screen too small problem in these two

modes, you can put this program in your AUTOEXEC.BAT and run SVGA ON

to generate a new C:\OS2\SVGADATA.PMI file.

OS2LDR, VSVGA.SYS, SVGA.EXE are video fixes.  They should

replace C:\OS2LDR, C:\OS2\MDOS\VSVGA.SYS and C:\OS2\SVGA.EXE.

PATCH256.EXE is a patch for all 256 color drivers.  Those

Trident driver users who experience screen trash problems during DOS

full screen to PM or OS/2 full screen to PM switching, this patch

will cause the PM desktop to refresh automatically, therefore removes

trash from the screen.  The patch applies only to 256 color drivers

including: T640.DLL, T768.DLL, T800.DLL, T1024.DLL, T8514.DLL, T640V.DLL,

T640V512.DLL and T800V.DLL.  To patch the above drivers, save a copy of

the original drivers, then run PATCHTRD.EXE from OS/2 session on each

of them.  For example,

PATCHTRD T800.DLL

will modify T800.DLL.  If screen trash problem does not occur on your

system you don't need to apply this patch.

8900BFIX.COM is a DOS TSR program which might be used to fix

certain screen-trash problem on some Trident OEM 8900B cards.  These

problems happen because current SVGA VDD driver (VSVGA.SYS) does not

correctly handle the hardware state save/restore.  These problems do

not happen on most cards. But if they do occur on your system, you can

try the 8900BFIX.COM. A convenient way to apply this fix automatically

when a DOS session is initiated is to put it into you AUTOEXEC.BAT.

BVH driver support, which is a nice feature with our 1.x drivers,

is no longer included in this package since OS/2 2.0 has build-in Trident

SuperVGA support.

PMFIX for 1.x should not be executed under 2.0.

OS2CRTC, which is used to adjust video parameters for 1.2/1.3

OS/2 drivers is not available at this time.  However, there is an

alternative.  See descriptions in "Appendices" below.

Installation:

-------------

0. Install OS/2 2.0 GA version with TVGA card on system

^^^^^^^^^^^^^^^^^^^^^^^^

(The following descriptions assume the startup drive is C:)

1. Run TRIDENT.EXE in DOS full screen session to determine

if the video card is Trident.  Trident PM drivers supports

Trident TVGA cards only

2. Make a backup copy of the following OS/2 drivers:

C:\OS2LDR  (need to run ATTRIB -S -H -R OS2LDR first)

C:\OS2\MDOS\VSVGA.SYS

C:\OS2\SVGA.EXE

C:\OS2\DLL\DISPLAY.DLL

(PM display driver)

C:\OS2\MDOS\WINOS2\SYSTEM\VGA.DRV

(WINOS2 full screen driver)

C:\OS2\MDOS\WINOS2\SYSTEM\SWINVGA.DRV

(WINOS2 seamless driver)

We recommend that the backup be put on a floppy

3. Unzip (*.ZIP) the Trident drivers/utilities/fixes to be

installed

4. Install the new OS2LDR, VSVGA.SYS and SVGA.EXE:

COPY OS2LDR C:\OS2LDR

COPY VSVGA.SYS C:\OS2\MDOS\VSVGA.SYS

COPY SVGA.EXE C:\OS2\SVGA.EXE

5. Go to a DOS Full Screen session, run system command

^^^^^^^^^^^^^^^^^^^^^^^

SVGA ON

A file called SVGADATA.PMI will be generated in C:\OS2

Save this file on a floppy disk

6. Switch to OS/2 protected mode session and start

DDINSTAL

7. Follow the instructions to finish the rest of the procedures

8. You need to shut down and re-boot the system before the

^^^^^^^^^^^^^^^^^^

driver takes into effect

9. Before installing T768.DLL, T640V512.DLL, T640V.DLL and

T800V.DLL, read the special discussions below

10.Copy SMODE.EXE, DESKMAN.EXE, DESKDLL.DLL etc to a directory

on you OS/2 PATH (eg. C:\OS2\APPS)

Appendices:

----------

1.  Files in the package:

.Zipped format:

W800     ZIP

W1024    ZIP

W8514    ZIP

T640     ZIP

T768     ZIP

T800     ZIP

T1024    ZIP

T8514    ZIP

T640V512 ZIP

T640V    ZIP

T800V    ZIP

UTILS    ZIP

FIXES    ZIP

README   EXE

.Unzipped format:

README   DOC    -- Read this first

W800     DLL    -- 800x600-16 color PM driver

W1024    DLL    -- 1024x768-16 color PM driver

T640     DLL    -- 640x480-256 color PM driver

T768     DLL    -- 768x1024-256 color PM driver

T800     DLL    -- 800x600-256 color PM driver

T1024    DLL    -- 1024x768-256 color PM driver

T8514    DLL    -- 1024x768-256 color PM driver

with 8514 compatible resources

T640V512 DLL    -- 640x480-256 color PM driver

with 1024x512 virtual desktop

T640V    DLL    -- 640x480-256 color PM driver

with 1024x1024 virtual desktop

T800V    DLL    -- 800x600-256 color PM driver

with 1024x1024 virtual desktop

W800SL   DRV    -- 800x600-16 seamless WINOS2 driver

W800     DRV    -- 800x600-16 full screen WinOS2 driver

W1024C   DRV    -- 1024x768-16 full screen WinOS2 driver

T640C    DRV    -- 640x480-256 full screen WinOS2 driver

T800C    DRV    -- 800x600-256 full screen WinOS2 driver

T1024C   DRV    -- 1024x768-256 full screen WinOS2 driver

T1024FC  DRV    -- 1024x768-256 full screen (large font)

W800     DDP    -- Device Driver Profile (for DDINSTAL)

W1024    DDP

T640     DDP

T768     DDP

T800     DDP

T1024    DDP

T8514    DDP

T640V512 DDP

T640V    DDP

T800V    DDP

TRIDENT  EXE    -- Trident TVGA chipset finder

DESKMAN  EXE    -- Desktop Manager

DESKDLL  DLL    -- used by DESKMAN.EXE

DISPINFO EXE    -- WinOS2 display info

SMODE    EXE    -- Set video mode for OS/2 and DOS

POLARITY COM    -- Change sync polarity for some displays

OS2LDR          -- OS/2 loader fix

VSVGA    SYS    -- VSVGA.SYS fix

SVGA     EXE    -- SVGA.EXE fix

PATCH256 EXE    -- fix 256 color desktop trash

8900BFIX COM    -- fix some 8900B problems

2.  On board video memory requirements (8900/9000)

driver        resolution      min. mem (KB)   chipset

W800.DLL      800x600 -16          256        8900/9000

W1024.DLL     1024x768-16          512        8900/9000

T640.DLL      640x480-256          512        8900/9000

T768.DLL      768x1024-256         1024       8900

T800.DLL      800x600-256          1024       8900

T1024.DLL     1024x768-256         1024       8900

T8514.DLL     1024x768-256         1024       8900

T640V512.DLL  640x480-256 (1Kx.5K) 512        8900/9000

T640V.DLL     640x480-256 (1Kx1K)  1024       8900

T800V.DLL     800x600-256 (1Kx1K)  1024       8900

Fail to comply with the above requirements may cause

system hangup during OS/2 boot.  In case this happens,

use a bootup diskette to boot from drive A, restore OS/2

default DISPLAY.DLL driver (the saved one), then boot

OS/2 again.

3.  Some useful tips for trouble shooting

. Use TVGA 8900C/9000 chips or newer.

8900B chip works in most cases.

. Use OS/2 2.0 GA version only.

OS/2 installation should be done after the install-

ation of TVGA card.

. Do not forget to backup original OS/2 drivers

. Do not forget to run SVGA ON system command from

DOS FULL SCREEN session.

. Alt-F1 on system boot time will erase SVGADATA.PMI

Back up the .PMI files before trying Alt-F1

. Make sure your TVGA board has enough memory for

the designated PM display mode.

. Expierenced users may choose to manually copy the

driver files to \OS2\DLL.  Make sure DISPLAY.DLL and

Seamless WINOS2 drivers use the same resolution

and color.  Full screen WINOS2 driver may use

different display mode.

. Whenever the TVGA board configuration is changed,

such as adding or removing display RAM, setting

jumpers, you need to run SVGA ON again to replace

.PMI file.  The best way for this is to run SVGA OFF,

restore the original VGA (640x480-16) DISPLAY.DLL,

erase old .PMI file first.  Change the hardware

configuration.  Then boot and run SVGA ON from DOS

full screen session.  Install the SVGA driver

and reboot.

. T768.DLL (portrait mode) needs a special .PMI

entry that currently can not be generated by

SVGA.EXE.  If you have problem installing this

driver, contact our technical support.

. If abnormal display occurs when running apps,

please try to duplicate the problem.  You may

also want to try the same apps under the original

VGA DISPLAY.DLL which comes with your OS/2.  If the

problem is still there, there is a high chance

that this is an application's bug.  Otherwise,

please report the problem to our technical

support.

. Bugs can usually be fixed within days.  But for

stability, the drivers may not be available to

the public immediately until the next major driver

release.  Private build can be obtained on individual

bases for evaluation.

. If you cannot get TVGA driver for WINOS2 working,

check to see if the driver is compressed.  A regular

size (just like the W800SL.DRV) is about 73000 bytes.

If yours is considerably smaller, you need to uncompress

it first.  Try the EXPAND.EXE program in GA Disk 9 to

uncompress the .DRV file.

4. Advanced topic: adjusting video parameters

In most cases, TVGA card matches display monitors

perfectly. However, some monitors cannot sync under

default video parameter settings.  Some can sync but

the picture shifted to one side of the screen.  Other

monitors support high refresh rates, that reduces

flickering (like 70 Hz vertical frequency).  To get

the best display result, you may want to go through

the following procedures to adjust video parameters.

Unless this is necessary, and you fully understand

how this works, we recommend that you skip this

operation.

. Boot OS/2 with the default DISPLAY.DLL (640x480-16).

. Use the TVGACRTC.EXE utility (on TVGA Driver/Utility

Diskettes, also available from Trident BBS) under OS/2

DOS FULL SCREEN.

It will generate TVGACRTC.SYS file and insert a line

DEVICE=path\TVGACRTC.SYS

in the CONFIG.SYS.

. Shutdown OS/2 and reboot the system.

. Open a DOS full screen and run

SVGA ON

This will generate a "corrected" .PMI file.

. Remove DEVICE=TVGACRTC.SYS statement from CONFIG.SYS

. Shutdown OS/2 and boot again.

5. Advanced topic: Vertual Desktop Display Driver

Virtual Desktop Drivers (T640V512.DLL, T640V.DLL and

T800V.DLL) are a new type of OS/2 display drivers.

The driver enables user to work on a larger desktop

which cannot be supported directly by the video display

hardware.  For example, your VGA monitor does not support

high resolution, or your VGA adapter is not able to set

video resolution higher than 1024x768 256 color.  Also on

a smaller monitor (say 14"), you may not want to use the

highest resolution mode but still want a bigger dimension

desktop.

Typical 256 color Virtual Desktop dimensions on TVGA

chipset are 1024x1024 for 1MB RAM, 1024x512 for 512KB

RAM.  Larger sizes may be supported on future TVGA

chipsets.

Since the physical screen (the viewport) is smaller than

the virtual desktop, there must be a way to move the

viewport around the desktop to display different parts

of the desktop.  Current driver design allows the user

to move mouse pointer beyond the boundary of the viewport,

to the direction of the part of desktop to be viewed.

The display driver will follow the mouse pointer and

adjust the position of viewport accordingly.

Some users may not like Virtual Desktop Drivers, for

various reasons.  They can use the T640.DLL or T800.DLL

instead.

To install T640V512.DLL, T640V.DLL or T800V.DLL, you need

to edit the .PMI file.  Make sure that you understand the

structure of .PMI file before editing it.  First make a

copy of the current .PMI file (in case you want to go back

to T640.DLL or T800.DLL).  Then use an editor to open

C:\OS2\SVGADATA.PMI.

If you are installing T640V512.DLL, find the mode entry

for 640x480-256 color mode from SVGADATA.PMI file:

[comment]

Graphics Mode: 640 x 480 x 256 colours.

and change the [modeinfo] section to:

(Be careful not to change any other lines in the file!)

[ModeInfo]

ModeAttributes   = 0x18

BytesPerScanLine = 80

XResolution      = 1024

YResolution      = 512

TextRows         = 30

BitsPerPixel     = 8

NumberOfPlanes   = 1

PageLength       = 524288

SaveSize         = 524288

InterlaceMode    = 0

BufferAddress    = 0x0a0000

All other steps are similar to installing T640.DLL.

If you are installing T640V.DLL, find the mode entry

for 640x480-256 color mode from SVGADATA.PMI file:

[comment]

Graphics Mode: 640 x 480 x 256 colours.

and change the [modeinfo] section to:

[ModeInfo]

ModeAttributes   = 0x18

BytesPerScanLine = 80

XResolution      = 1024

YResolution      = 1024

TextRows         = 30

BitsPerPixel     = 8

NumberOfPlanes   = 1

PageLength       = 1048576

SaveSize         = 1048576

InterlaceMode    = 0

BufferAddress    = 0x0a0000

All other steps are similar to installing T640.DLL.

If you are going to install T800V.DLL, find the mode

entry for 800x600-256 color mode from SVGADATA.PMI file:

[comment]

Graphics Mode: 800 x 600 x 256 colours.

and change the [modeinfo] section to:

[ModeInfo]

ModeAttributes   = 0x18

BytesPerScanLine = 100

XResolution      = 1024

YResolution      = 1024

TextRows         = 37

BitsPerPixel     = 8

NumberOfPlanes   = 1

PageLength       = 1048576

SaveSize         = 1048576

InterlaceMode    = 0

BufferAddress    = 0x0a0000

All other steps are similar to installing T800.DLL.

6. Zoomable Virtual Desktop

Zoom feature for Virtual Desktop is designed to increase

the usability of the drivers and the productivity of

users. Since the Workplace Shell has no knowledge about

the difference of virual desktop and physical screen

resolution, the original virtual desktop drivers was

inconvenient when a window is bigger than the physical

screen size or too small that could hide itself beyond

the visible area.  With the new virtual desktop drivers

and the DESKMAN utility, this problem is greatly

alleviated.  DESKMAN does three jobs.  First it shows

desktop/driver informations so that the user always

knows what drivers he/she is using and the desktop , screen,

icon, mouse dimensions and number of colors.  Second,

the driver can disable desktop panning so that moving the

mouse pointer out of screen will not cause the visible

desktop area to slide away.  Whenever you want to focus

on working on a desktop area, press the SCROLL LOCK key

will lock the screen.  Press that key again undo the

locking.  You also have the option of temperarily reverse

current locking state by press and hold both SHIFT and

CTRL keys.  Try press and hold these keys and move the

mouse pointer to where you want and release both keys.

Third, DESKMAN can zoom in the big desktop into the

whole screen area so that the user is able to get overview

of the entire desktop.  The virtual desktop driver does

this by shrink itself into the screen.  Typically a 1024

by 1024 desktop will shrink to 512 by 512, and 1024 by

512 to 512 by 512.  Screen data is not lost and can be

instantly restored.  User can even work on most functions

when screen is zoom-in'ed.  One usage for this feature is

to find a small off-screen window by zoom in the desktop,

move mouse to the window of interest the zoom back to

normal size.  The hot keys for zoom-in are CTRL-ALT.

By press and hold CTRL-ALT, the desktop is zoomed in.

Release CTRL-ALT to undo it.

To obtain zoomable feature, DESKMAN.EXE should run first.

Since DESKDLL.DLL is loaded at runtime by DESKMAN.EXE,

it should be in the same directory as DESKMAN.EXE or on

your OS/2 LIBPATH.  With DESKMAN stays 'resident', the

zoomable features are always available with one keystroke

away!  Typically, after invoking DESKMAN, you just minimize

(hide) it so that it will not shown on desktop.  We

recommend that you put it into the STARTUP folder (within

system folder) and make it 'minimized on startup'.

7. Questions and Answers

Q: How do I know my card is Trident ?

A: If your are using Impact III card, it is Trident.

To determine if the card is Trident OEM, four

ways you can try:

. The system boot up logo shows 'TRIDENT'

. Run TRIDENT.EXE utility in DOS full screen session

. Run SVGA STATUS from DOS full screen

. The big square chip on the card shows 'Trident'

Q: I just installed Trident drivers on my TVGA system,

and the system hangs on the first reboot.  Give

me some tips.

A: Here is a check list:

. C:\OS2\SVGADATA.PMI exists and is about 17217

bytes long

. CONFIG.SYS contains the following three lines:

IOPL = YES                     (OS/2 default)

SET VIO_VGA=(BVHVGA,BVHSVGA)

DEVICE=C:\OS2\MDOS\VSVGA.SYS

Q: Nobody seems can get Trident drivers working

on my SVGA card, what's wrong ?

A: Is the card using Trident 8900/9000 chipset ?

Q: What do BVHSVGA.DLL and VSVGA.SYS do ?

A: From SVGA display driver point of view, BVH handles

OS/2 sessions and VSVGA handles multiple virtual

DOS sessions.

Q: I have a problem with my 8900B when opening a DOS

full screen. How can I solve the problem ?

A: Try 8900BFIX. This is a temporary solution and

might not fix your specific problem. Only newer

VSVGA.SYS can permanently fix this problem.

Q: My desktop is trashed during DOS or OS/2 full screen

to PM switch.  Is this a bug in the DISPLAY.DLL ?

A: This generally indicates some incorrect operations

in VSVGA.SYS or BVHSVGA.DLL.  The problem happens

mostly when using 256 color drivers and 8900B chips.

You can always apply the PATCH256.EXE to cure it

completely.

Q: GIF's on my 256 color desktop seems not as colorful

as what I expected, any comment ?

A: It depends on the image viewer you use.  Trident

256 color drivers support realizable color palette

manipulation.  This feature is undocumented in

OS/2 2.0, although available in OS/2 1.x. To find

if a display driver supports realizable color

palette, use

DevQueryCaps

and check the CAPS_COLOR_TABLE_SUPPORT entry. An 8

indicates it supports 256 colors. Then you can

call the following calls/undocumented calls:

GpiCreateLogColorTable

GpiRealizeColorTable

GpiUnrealizeColorTable

and be sure to include the following two lines at

the begin in of your program:

BOOL    APIENTRY16 GpiRealizeColorTable(HPS);

BOOL    APIENTRY16 GpiUnrealizeColorTable(HPS);

Also you need to do is to put these lines in your

.DEF file:

IMPORTS

GPIREALIZECOLORTABLE    = PMGPI.100

GPIUNREALIZECOLORTABLE  = PMGPI.101

Write your true 256 color image viewers and have

fun!

Q: I tried all your 256 color PM drivers, none of them

would allow me to run my 16 color graphics apps

from a DOS window. I have no problem with the OS/2

VGA display driver. Does this mean your drivers are

buggy ?

A: No. Nobody who does VGA compatible 256 color drivers

would ever try to think of supporting such features.

There is a substantial difference between 16 color

and 256 color modes in term of hardware structures.

VGA compatible 16 color modes use planar memory layout,

whereas VGA compatible 256 color modes use packed

pixel memory layout. In an OS/2 DOS windowed session,

all direct hardware access are trapped/intercepted

by the OS and the hardware operations are emulated.

This is called virtualization. Text modes are the

most easy modes for emulattion, whether the hardware

is in 16 or 256 color graphics mode. It is also

practical for software to emulate 256 color modes

when hardware is in 16 color mode. This is the case

when you use the VGA display driver and have a 256

color game running in a DOS window. But on the other

hand, trapping and fully emulating 16 color modes

with 256 color hardware settings is in practical

if not impossible. Even in the future OS/2 release

I would doubt this limitation can be overcome.

The solution is to run all DOS graphics apps from

DOS full screen if you are using 256 color desktop

driver or you will be forced to do so.

Support for 256 color windowed DOS graphics in a

256 color desktop could be included in future OS/2

releases.

Q: I need to install a dual monitor system, but hasn't

been successful in getting my mono card to work

with my TVGA card.  Is it possible for this setup ?

A: Sure.  But wait a minute.  We has found that OS/2

2.0 installation incorrectly modified the CONFIG.SYS

file when SVGA is used.  Before installing mono

card, OS/2 has this statement in CONFIG.SYS:

SET VIO_VGA=DEVICE(BVHVGA,BVHSVGA)

But after you run mono instalation, you will find

the statement has been changed to:

SET VIO_VGA=DEVICE(BVHSVGA,BVHVGA)

which will cause the next reboot to fail.  You

need to manually edit CONFIG.SYS immediately after

mono installation and then reboot machine.

We also found that the OS2LDR has a problem when

both TVGA and MDA are installed.  This can be solved

by using TVGA BIOS 3.x.

Q: 800x600 drivers shrink my screen to half height.

Can you help me get rid of this?

A: Put the POLARITY.COM TSR in AUTOEXEC.BAT, and open

a DOS full screen. Run SMODE 5B and see if the

problem is solved.  If yes, run SVGA ON under this

session and the corrected .PMI file will be generated.

Restart your machine.

Q: When running in a DOS windowed session, my VGA

graphics apps are always suspended, why ? I am

using a 256 color driver.

A: Under 256 color PM, it is too expensive for the

virtual DOS machine to emulate 16 color hardware

operations.  Run the apps under DOS full screen.

Q: My DOS or OS/2 full screen is trashed when I start

a full screen session.  Am I the only one who

have this problem?

A: This problem does not happen to most TVGA owners,

but does occur to some. It has nothing to do with

the desktop driver (DISPLAY.DLL) but definitely

indicates the current VSVGA.SYS or BVHSVGA.DLL

do not handle some cards correctly.

Q: My monitor supports higher vertical frequency

(70Hz).  Does these drivers allow me to utilize

this feature ?

A: Yes.  The way that Trident drivers are designed

makes it especially flexible.  You don't need

to download a 70 or 64 Hz driver separately.

What you need to do is to use the TVGACRTC

utility mentioned above to adjust the monitor

display to the best you want.  Don't know if

your monitor can support higher vertical

frequency ?  Try TVGACRTC and find it out

yourself.

Q: Are there any neat tricks for OS/2 SVGA ?

A: Find them yourself and write 'Undocumented OS/2'.

Practice:    SVGA.EXE

Use DEBUG to search 'STATUS' string, you will

find SVGA.EXE supports more options other than

ON/OFF:

SVGA STATUS

SVGA WHO

SVGA ON DOS

Try these and find out what they mean.

WARNING: Improper use of undocumented features

may cause unpredictable result.

Q: I installed W800.DLL, do I have to use W800.DRV

for WinOS2 ?

A: No. You can choose any WinOS2 driver (same as

Windows 3.0 driver) for your full screen session.

However, seamless DISPLAY.DLL and SWINVGA.DRV

have to be in the same mode.  Thus W800SL.DRV

can only be used with W800.DLL.

Q: I used T640V.DLL.  I sometimes lose where I am

on the 1024x1024 Virtual Desktop.  Any ideas ?

A: I usually place a desktop picture at the center

of my desktop (not Tiled or Scaled) so that I can

always refer to the picture to know where I am.

With this release use the ZOOMABLE feature to

increase virtual desktop usability.

Q: Does Trident has any newer high performance chips

available ?  What about future OS/2 support ?

A: Yes, we have several new chips coming out soon.

They are compatible with 8900C.  OS/2 and Trident

drivers has been tested and found working very

well on them.  Send your inquiries to our technical

support for more info.  We will continue support

our customers by providing better drivers.

Q: Where can I get additional OS/2 info ?

A: To get info from IBM, call 1-800-3IBMOS2.

If you have Internet account, join the following

news group:

comp.os.os2.misc

comp.os.os2.apps

comp.os.os2.programmer

etc.

or ftp from:

hobbes.nmsu.edu

If you do not have Internet access, try the BBSes

listed above.

*******************************

*  Trident Microsystems, Inc. *

*     205 Ravendale Drive     *

*   Mountain View, CA 94043   *

*******************************

FAX:    (415) 691-9265  Attn: OS/2 Driver Support

BBS:    (415) 691-1016  (Original distribution site for this package)

File location: File Library\OS2-20

8 lines now! 9600/2400 bps

Parity: NONE

Data:   8

Stop:   1

CompuServe CIS: 71141,3261

* This package is provided by Trident Microsystems, Inc.  It can be

freely distributed to other electronic media.